Amadeus Peripheral Manager

- User Guide -

Index

1	Preface	4

2	Introduction	4

3	Pre-requisites	5

4	Devices	5

5	Device local sharing	6

6	Device locking	6

7	1A-PM features and other CUTE PMs	7

8	Configuration	8

8.1	Print Emulator	8

8.2	Peripheral Manager	10

8.2.1	Configuration Options	10

8.2.2	Configuration Format	18

9	Deployment	18

9.1	Print Emulator	18

9.2	Peripheral Manager	19

9.2.1	Deliverables	19

9.2.2	Packaging	19

9.2.3	Installing/Uninstalling 1A-PM	19

9.2.4	Custom logging options (optional)	23

9.2.5	Custom configuration file location (optional)	23

9.2.6	Start delay (optional)	24

9.2.7	Custom TCP port number (optional)	24

9.2.8	Workstation ID (optional)	24

9.2.9	Disable the mode command	24

9.2.10	Enforcing 1A-PM version (optional)	25

9.2.11	Starting/Stopping 1A-PM	25

10	Conclusion	25

11	Glossary	27

12	Appendix	27

12.1	How to install the generic text spooler?	27

12.1.1	Generic printer attached to the LPT port	27

12.1.2	Generic printer attached to an USB port	28

Document control

| Security level | Airline customers only |  |  |  |
| --- | --- | --- | --- | --- |
| Company | Amadeus IT Group S.A. |  |  |  |
| Department | DEV-AIR-CMD-AMD-JAP |  |  |  |
| Author | Sebastien Taghite |  |  |  |
| Reviewed by | Thibault Deman |  | Date | 30/08/2010 |
| Approved by |  |  | Date |  |
| Version | Date | Change | Comment | By |
| 1.0 | 30/08/2010 | Unified version | Initial | Sebastien Taghite |

# Preface

The aim of this document is to describe the usage and configuration of Amadeus Peripheral Manager (1A-PM) for native DCS and RES platforms, known also as Non-CUTE platforms. It is assumed herein that the reader is familiar with general aspects of Amadeus devices handling solution.

# Introduction

In order to close the gap between CUTE and Non-CUTE platforms, Amadeus provides its clients with a CUTE-like peripheral manager for native platforms. 1A-PM runs on target workstations as a Windows™ service, which listens on a configurable TCP port for client requests. 1A-PM is designed with extensibility and configurability in mind, in order to cope with native platform heterogeneity.

Amadeus peripheral manager plays two main roles:

###### I/O port adapter

A user application should be able to use any supported physical device without having to know anything about its actual physical port and low-level communication protocol. In other words, 1A-PM acts as a driver for all underlying supported serial devices (and Windows spooler), thus exposing them to IP clients on airport LAN.

###### Device access arbitrator

Multiple clients should be able to transparently share any supported physical device. Device sharing implies that 1A-PM implements device locking, and possibly context management, to ensure that shared devices are accessed in a consistent way.

Figure 1 below provides an overview of how 1A-PM is used in production:

Amadeus Peripheral Manager (1A-PM)

User Application (GUI)

Amadeus Print Emulator (PE)

TCP/IP

MSR

OCR

LSR

Airlines’ input devices

ATB

BTP

DGR

Airlines’ output devices

PRT

General purpose printers

1A-PM configuration file (XML)

Read

Read

Optional

PE configuration file (Java properties)

Figure 1 - 1A-PM overview

There are few important things that we should point out here:

- 1A-PM can be accessed only through Amadeus Print Emulator, since its API is not currently published. Besides, 1A-PM and PE + GUI are not necessarily running on the same workstation.

- 1A-PM learns its configuration from an XML file, which must follow Amadeus conventions (i.e. XML schema). This file can be either local or remote, as long as it appears to 1A-PM as a local file (e.g. mapped Windows™ network drive).

- PE can be configured to locate target 1A-PM instance(s) anywhere on the network and/or select any list of target devices, by using an optional Java™ properties configuration file.

# Pre-requisites

All target platforms must satisfy the following requirements:

- TCP/IP communication between PE and 1A-PM, possibly in loop-back.

- Devices must be plugged on the workstation’s serial port, or accessible via the Windows spooler, or plugged to the airport LAN.

- Currently estimated minimum requirements for target workstations are:

  - Windows™ NT4, 2000 or XP.

  - Pentium 200 MHz

  - 64 MB of RAM

  - 10 Mbps Ethernet network interface

  - 100 MB of free hard disk

# Devices

We currently support the following device types on native platforms:

- ATB (a.k.a BPP)

- BTP

- PRT (a.k.a. GPP or DCP)

- DGR (a.k.a. BGR)

- MSR

- OCR

- LSR

- BCP (ATB dedicated to boarding passes

- TKP (ATB dedicated to tickets)

Notice that MSR, OCR and LSR can be either standalone (i.e. use separate serial ports) or share the same serial port as in the case of Access keyboards.

Notice that PRT printers can be targeted via the Windows spooler (see section 8.2.1).

The exhaustive list of all supported models and firmware versions is to be found in the devices catalogue that is signed with each airline individually.

# Device local sharing

In addition to local devices, our printing solution allows user applications to access also remote devices, as long as they are handled by an accessible Amadeus or CUTE peripheral manager: this is called local sharing.

In local sharing, it is the print emulator that routes application’s requests to the right peripheral manager, and ultimately to the right device. For example, in Figure 2 below, the user application on workstation 1 can access, in addition to local ATB1 and BTP1, a remote generic printer (PRT1) on workstation 2.

Similarly to CUTE peripheral managers, Print Emulator relies on our non-CUTE Java API library to interface with local and remote 1A-PM instances, thus shielding PE from low-level TCP/IP processing.

Workstation 2

Workstation 1

User Application (GUI)

Print Emulator (PE)

Non-CUTE Java API

TCP/IP

TCP/IP

ATB1

BTP1

PRT1

1A Peripheral Manager

1A Peripheral Manager

Figure 2 – Local sharing on native platforms

# Device locking

1A-PM allows a client to lock a given device, in order to gain exclusive access for printing and unsolicited data input events.

We currently support the following lock options:

  - Locking of input devices (MSR, OCR and LSR) is pre-emptive and is associated to system focus: i.e. application in foreground can always grab the device lock.

  - Locking of other devices (ATB, BTP, DGR and PRT) expires after a configurable timeout (e.g. 60sec), in order to give other clients a chance to acquire such device lock.

Notice that when a sharable device such as ATB is not locked, all clients are able to issue print requests, and all unsolicited data input events are discarded.

# 1A-PM features and other CUTE PMs

Table 1 below describes the 1A-PM features compared to some other CUTE PMs.

|  | 1A-PM | RESA-PM | SITA-PM | ARINC-PM |
| --- | --- | --- | --- | --- |
| Sharing |  |  |  |  |
| Multi-context by airline? | No | Yes | Yes | Yes |
| Multi-context by application? | No | Yes | No | No |
| BGR shareable? | No | No | No | No |
| Concurrent Write |  |  |  |  |
| Serialization of calls by PM | Yes | Yes | Yes | Yes |
| Configurable time-out to wait device is free | Yes | Yes | Yes | No |
| Exclusive Lock |  |  |  |  |
| Lock | Yes | No | Yes | No |
| Locker receiving input events | Yes | N/A | Yes | N/A |
| Rule to receive input events? | Only if locked. | Last application that opened or written on device. | Locked (or first application that opened the device if not locked) | Last application that opened or written on device. |
| Acquire time-out? | Yes | N/A | Yes | N/A |
| Keep time-out? | In PM configuration | N/A | In PM configuration | N/A |
| Notification on lock loss? | Yes | N/A | Yes | N/A |
| Rule to receive input event on lock loss? | Event discarded | N/A | Event discarded (lock lost only if new application requested it) | N/A |
| Concurrent write possible on lock? | No | N/A | No | N/A |
| If no is there waiting of lock release? | Yes | N/A | N/A | N/A |
| Lock kept on read? | Yes | N/A | Yes | N/A |
| MSR, LSR and OCR focus |  |  |  |  |
| Shareable? | Yes | No | Yes | Yes |
| API to request focus? | Yes | N/A | Yes | Yes |
| Grab lock? | Yes | N/A | No | No |
| Explicit lock release? | Yes | N/A | No (close device) | Yes (or close device) |
| Keep time-out? | In PM configuration | N/A | In PM configuration | No |
| Lock loss notification | Yes | N/A | Yes | No |

Table 1 - 1A and CUTE PM features comparison table.

# Configuration

## Print Emulator

Similarly to CUTE platforms, PE does not generally require any configuration on non-CUTE platforms, because PE uses - by default - all available devices on local 1A-PM at TCP port 40000.

However, there are at least four cases, where a PE configuration file is needed:

- When local 1A-PM TCP port is different from default port (i.e. port ≠ 40000)

- When PE is targeting a remote 1A-PM (i.e. ip ≠ localhost)

- When PE uses only a subset of local devices

- When PE targets multiple 1A-PM instances (local sharing)

In all cases above, we use Java style property file to define IP/port for each target 1A-PM and select local/remote devices. The name of this file is passed to PE using the following command line option:

--pe_conf=<path of PE configuration file>

In general, we support two different and mutually exclusive configuration schemes for PE:

- “Use all local devices” scheme

In this scheme we simply indicate IP/port of the target 1A-PM. This is the default scheme: when no IP/port is specified, PE uses localhost and default port (i.e. 40000).

- “Explicit devices list” scheme

This scheme allows a given PE/GUI to indicate explicitly which local and/or remote devices are needed. The syntax used to identify a device uses the following regular expression:

<Device type> + <Device real index> + [‘/’ + IP +’:’ + Port [ + ’/’ + <Device local index>]]

Note1: If IP and port are not specified, we default to localhost and 40000.  However, this can be changed by setting a different IP and/or port for the default 1A-PM.

Note2: Since different devices can have the same type and index on different workstations, we must provide a mechanism to avoid naming collisions. This can be done in two ways:

  - Automatically

PE assigns automatically an appropriate local index to any duplicate device.

  - Manually

Users can configure the optional “device local index” field. In this case, care must be taken to avoid local index collisions, since PE logs a warning for any duplicate device that is manually configured, and ignores it.

The following examples are all valid expressions:

  - “ATB1”

Local ATB1 (i.e. IP = localhost and port = 40000)

  - “BTP1/10.0.0.1:40000”

Remote BTP1, called BTPx locally, where x is assigned dynamically by PE to avoid index collisions: i.e. x=1 if no other BTP1 exists or x=<last available index for BTP devices>.

  - “DGR1/ 10.0.0.2:40001/2”

Remote DGR1, called DGR2 locally.

For convenience, we can select all devices of a peripheral manager using the following regular expression:

‘*’ + [‘/’ + IP +’:’ + Port]

Exemples:

  - “*/10.0.0.1:40001”

Refers to all devices handled by the peripheral manager at 10.0.0.1 on port 40001

  - “*”

Refers to all local devices (i.e. IP = localhost and port = 40000)

Table 2 below summarizes our most up-to-date PE configuration file content and format for Non-CUTE platforms; future changes (if any) will be documented in subsequent versions of this document:

| Description | Java property name | Optional? | Default |
| --- | --- | --- | --- |
| Default 1A-PM IP | amadeus.printemulator.platforms.noncute.default.pm.ip | Yes | localhost |
| Indicates the IP address or DNS name of the default target peripheral manager. Use this option only when talking to a remote 1A-PM. Example: amadeus.printemulator.platforms.noncute.default.pm.ip =10.0.0.1 |  |  |  |
| Default 1A-PM Port | amadeus.printemulator.platforms.noncute.default.pm.port | Yes | 40000 |
| Indicates the TCP port number of the default target peripheral manager. Use this option only when default port is not used. Example: amadeus.printemulator.platforms.noncute.default.pm.port =55555 |  |  |  |
| Devices list | amadeus.printemulator.platforms.noncute.device.list | Yes | * |
| Contains a comma-separated list of all device names to be used by the print emulator. All device names use the defined format above. Use this only when you need to choose a specific set of devices, or when you need to access one or many remote devices. Example 0: amadeus.printemulator.platforms.noncute.device.list=DGR1,PRT1 In this example PE is configured to use only local DGR1 and local PRT1. Example 1: amadeus.printemulator.platforms.noncute.device.list=ATB1,ATB1/10.0.0.2:40000/2,PRT1/10.0.0.3:40000 In this example PE is configured to use: - Local ATB1 - Remote ATB1, known locally as ATB2 - Remote PRT1, also known locally as PRT1 Example 2: amadeus.printemulator.platforms.noncute.device.list= *, PRT1/10.0.0.3:40000 In this example PE is configured to use: - All local devices - Plus a remote PRT1 (that will be renamed to something else if a local PRT1 is found). Example 3: amadeus.printemulator.platforms.noncute.device.list = */10.0.0.2:50000 In this example PE is configured to use all devices on the specified workstation. Notice that the same behaviour can also be obtained using the following parameters: amadeus.printemulator.platforms.noncute.default.pm.ip=10.0.0.2 amadeus.printemulator.platforms.noncute.default.pm.port =50000 |  |  |  |

Table 2 –PE configuration options on Non-CUTE platforms

As we can see, the suggested configuration format supports the following features:

  - Using all or a sub-set of local devices

  - Using all or a sub-set of the devices of any remote workstation

  - Manual/automatic device renaming to avoid index collisions

Notice that we do not support advanced regular expressions to select very specific devices subsets such as “all ATB devices” or “all devices except BTP1”, because we believe this is error-prone and unnecessary. Indeed, airlines usually configure all (or most of) their workstations automatically in a centralized manner; therefore it is worth the effort to write explicitly the entire device list when needed. Besides, we believe that in most cases PE will not require any configuration at all, because very often user applications only need to access local devices.

## Peripheral Manager

We aim at making our native peripheral manager both thin and device-independent, while still providing a reasonable level of service, such as device-specific sharing and locking behaviours. Therefore, we have decided to export as much of device knowledge as possible outside the peripheral manager, as follows:

- All data format and application protocol information are entrusted to the print emulator and the host.

- All device capability and connectivity information are located in a local configuration file, which is loaded by the peripheral manager at start-up.

### Configuration Options

For each device the configuration file contains three types of information:

- Device identification

This section contains information which is specific to each device such as device type, index and vendor.

- Device capability

The peripheral manager uses some of this information to know which operations can be performed on a given device (e.g. sharing, remote access, one-way vs. two-ways …). And some of it is forwarded to the print emulator, which in turn uses it to efficiently interact with the physical device (e.g. query logical status).

- Device connectivity

This section contains the physical port (e.g. COM1) and parameters (e.g. Baudrate, parity, stop bits …), as well as logical and data-link protocol information (e.g. maximum size of payload, type of checksum, Ack/Nack …).

Table 3 below represents the current list of needed parameters to our knowledge. Future changes (if any) will be documented in subsequent versions of this document:

Messages exchanged between the 1A-PM and devices have the following structures.

  - Outgoing block: i.e. from 1A-PM to device:

<STX>

Block Seq. number

Block

Header

Payload

<EOM>

Middle flag

EMBH

ETB for middle block

ETX for end of block

Middle/End Block Trailer

<ETX>/<ETB>

Block Checksum

<SOM>

Figure 3 – Outgoing block structure

  - Incoming block: i.e. from device to 1A-PM:

<STX>

Block Seq. number

Block

Header

Payload

<EOM>

EMBH

<ETB> for middle block

<ETX> for end of block

Block Trailer

<ETX>/<ETB>

Block Checksum

<SOD>

<EOD>

<SOD> and <EOD> are applicable to SITA compliant MSR/OCR/LSR keyboards only.

<SOM>

Figure 4 - Incoming block structure

Notice that all the fields above are not necessarily present in each block; instead each field can be activated or deactivated individually for each device in 1A-PM configuration file. In addition some combinations of the fields above are not compatible. Please refer to your device documentation to know which options need to be enabled.

Table 3 below describes the current list of all 1A-PM configuration options. Future changes will be documented in subsequent versions of this document. Table 3 refers to the block structures in Figure 3 and Figure 4 above for protocol and data-link options.

|  | Field |  |  |
| --- | --- | --- | --- |
|  | Name | Optional? | Description |
| Device Identification Options (All devices) | Device type | No | Three characters in capital letters, which indicate the type of the device. We currently support the following device types: ATB, BTP, DGR, PRT, MSR, LSR and OCR. |
|  | Device index | No | This is an unsigned integer starting from 1, which is used to uniquely identify a physical device among all other devices of the same type. This index is combined with the device type to form a unique ID (name) for this device (e.g. “ATB1”). |
|  | Brand | Yes | This field indicates the vendor of this device (e.g. “IER”). Default value is “N/A”. |
|  | Model | Yes | This field indicates the model of this device (e.g. “557”). Default value is “N/A”. |
|  | Firmware version | Yes | This field indicates the firmware version of this device. Default value is “N/A”. |
|  | Firmware release date | Yes | This field indicates the firmware release date of this device. Default value is “N/A”. |
|  | Mode | Yes | This field indicates the mode of this device (e.g. “Domestic”, “BTP Terminal”…). Default value is “N/A”. |
| Device Capability Options (All devices) | Is device sharable? | Yes | This field tells the peripheral manager whether this device is sharable or not. By default all devices are sharable, except boarding gate readers (DGR) which are not. |
|  | Authorize remote access to device? | Yes | This field tells the peripheral manager whether to accept or refuse remote PE connections to this device. Indeed, it may be functionally useful (or required for security reasons) to limit access for a given device to local PEs only. By default remote access is allowed for all devices, except MSR, OCR and LSR devices which are by default not accessible by remote users. |
|  | Is device bidirectional? | No | This field tells the peripheral manager and the print emulator, whether this device is replying (two-ways) or not (one-way). By default only ATB and DGR are bidirectional (two-ways). All other devices are considered one-way unless configured otherwise. |
|  | Device’s lock keep timeout | Yes | Indicates how long, in milliseconds, the lock of this device can be granted to a requesting PE. When this timeout expires, the lock is reclaimed by the peripheral manager, in order to avoid starvation of other clients. Default value for lock keep timeout is 60 seconds. Note: you can disable the keep-lock-timeout feature for a given device by setting it to a very big value. This is useful for instance for MSR/OCR/LSR whose lock acquisition/loss is linked to window focus. |
|  | List of all supported non-AEA commands | Yes | This field is mainly used to tell the print emulator which logical status commands are supported by this device. However, it can also be used for other non-AEA commands such as SM. Since QS can be implemented differently by two devices, we define a unique code name for each command. For example, we currently support the following commands: - SITA_QS refers to SITA’s QS command - RESA_QS refers to RESA’s QS command - ARINC_QS refers to ARINC’s s/ command - SITA_SM refers to SITA’s SM command - RESA_SM refers to RESA’s SM command - ARINC_SM refers to ARINC’s m/ command The field contains a comma-separated list of all supported command codes above. By default no such commands are supported. |
|  | AEA command prefix | Yes | This field can be used to indicate the prefix used by this device for AEA commands. Typical AEA command prefixes are: “AD;” and “AD:” This field must be configured explicitly for AEA devices when needed. |
|  | AEA reply prefix | Yes | Similarly this field can be used to indicate the prefix used by this device for AEA replies. Typical AEA reply prefixes are: “AR;” and “AR:” This field must be configured explicitly for AEA devices when needed. |
| Serial devices ONLY | Port name | No | The physical serial port name on which this device is plugged (e.g. “COM1”) |
|  | Baudrate | No | The serial baudrate value for this device. Notice that we accept any baudrate value, not only common RS232 baudrate values. |
|  | Data bits | No | The number of data bits in a serial block; We support all possible data-bits values. i.e. 5, 6, 7 and 8. |
|  | Stop bits | No | The number of stop bits. We support all possible stop bits values. I.e. 1, 1.5 and 2. |
|  | Parity | No | The parity type for this device. We support all possible parity values, as follows: - “N”  No parity - “E”  Even parity - “O”  Odd parity - “M”  Mark parity - “S”  Space parity |
|  | Flow control options | Yes | Indicates whether to use or not hardware and/or software flow control when communicating with this physical device. The actual parameters used to set flow control depend entirely on Win32 API. The latter uses the following fields: - Timeout = ON ¦ OFF - XON/XOFF = ON ¦ OFF - CTS handshaking = ON ¦ OFF - DSR handshaking = ON ¦ OFF - DSR sensitivity = ON ¦ OFF - DTR circuit = ON ¦ OFF ¦ HS - RTS circuit = ON ¦ OFF ¦ HS ¦ TG More information about the parameters above can be found on the MSDN. The default value is: - Timeout = OFF - XON/XOFF = OFF - CTS handshaking = ON - DSR handshaking = ON - DSR sensitivity = ON - DTR circuit = ON - RTS circuit = ON However, this value may change depending on collected information about low level connectivity parameters used by each airline devices on the field. |
| Spooler Device only | Spooler name | No | Indicates the value of the spooler name to open. Usually this is simply the local printer name or network printer name (e.g. \\10.0.01\my_printer) or a fully qualified network printer URL (e.g. http://10.0.0.1/printers/ my_printer /.printer). However for some network printers Windows Spooler uses the following specific naming convention: \\<Network Printer URL> e.g. \\ipp://my_ipp_server \my_printer Generally, for generic print, text data is sent to the printer. As a matter of fact, the generic text spooler should be installed for the used printer. To find out how to install the generic text spooler, see section 12.1. Note that the used printer has to natively support text data. If the printer does not natively support text, then “the direct print from client” option can be used. The print will be then delegated to the Print Emulator instead of the 1A-PM. The GDI will be used allowing conversion of text to the native language of the printer. Note that this option should be used only for laser printers (more generally for page oriented printers). With line oriented printers, some paper could be wasted because of the form feed. Finally notice that due to Windows Spooler limitations the physical status (online, offline …) of generic printers can not be notified to applications. Therefore such printers will always appear online. |
|  | Append form feed | Yes | The default value for this option is false. In this case, a form feed (0x0C) is appended at the end of each print in order to print the page. This option is useful with laser printers which print the page only when the page is full of chars or when a form feed char is received. Standard generic text printers do not require this option set to true since data is printed when a line is received. |
|  | Retry open | Yes | This property indicates if the spooler need to be re-open after that an error different than “invalid spooler name” is encountered. It will be re-open 2 times by default. This value can be overridden in the configuration file. |
|  | Open-Close at printing | Yes | The default value for this option is false. If this option is set to true, then the spooler is open just before the print and closed just after print is performed. The normal flow (with option set to false) is: spooler is opened when 1a-pm is started and closed when 1a-pm is stopped |
|  | Rights | Yes | Specify the access rights needed to the printer. The access rights can be one of the following: - None: no rights used - Use: to perform basic printing operations - Admin: to perform administrative tasks - All: to perform all administrative tasks and basic printing operations except for SYNCHRONIZE |
|  | Print from client | Yes | The default value for this option is false. The print from client allows the print to be performed by the PrintEmulator instead of the 1a-pm. All the previous spooler options do not apply in the “print from client” mode. This mode can be used when permission problems are encountered or when the printer does not natively support raw text (the PrintEmulator will forward the print request to the windows driver which will perform the conversion). |
|  | Font name | Yes | Default value is “monospaced”. This option only applies in print from client mode. |
|  | Font size | Yes | Default value is “9”. This option only applies in print from client mode. Modifying this option value will change the maximum number of chars printed in one page. |
| Device Protocol Options (All devices except Generic Printers ) | ACK | Yes | Indicates the value of ACK control byte and whether to use it or not. Default value for ACK is 0x06. ACK is not used by default. |
|  | NAK | Yes | Indicates the value of NAK control byte and whether to use it or not. Default value for NAK is 0x15. NAK is not used by default. |
|  | STX | Yes | Indicates the value of STX control byte and whether to use it or not. Default value for STX is 0x02. STX is not used by default. |
|  | ETX | Yes | Indicates the value of ETX control byte and whether to use it or not. Default value for ETX is 0x03. ETX is not used by default. |
|  | ETB | Yes | Indicates the value of ETB control byte and whether to use it or not. Default value for ETB is 0x17. ETB is not used by default. |
|  | SOM | Yes | Indicates the value of SOM control byte and whether to use it or not. Default value for SOM is 0x80. SOM is not used by default. |
|  | EOM | Yes | Indicates the value of EOM control byte and whether to use it or not. Default value for EOM is 0xFF. EOM is not used by default. |
|  | DLE options | Yes | Indicates the value of DLE control byte and the list of bytes that need to be escaped. This list can be specified in one of two forms: - As a range of hexadecimal bytes e.g. “00-20”  {0x00, 0x01, 0x02 …, 0x20} - As a list of comma-separated hexadecimal bytes e.g. “02,10,FF”  {0x02, 0x10, 0xFF} Any character in payload which belongs to this list (including DLE itself) will be preceded by a DLE character. Default value for DLE is 0x10. All bytes between 0x00 and 0x20 are escaped by default. |
|  | ENQ | Yes | Indicates the value of ENQ control byte and whether to use it or not. Default value for ENQ is 0x18. ENQ is not used by default. |
|  | CAN | Yes | Indicates the value of CAN control byte and whether to use it or not. Default value for CAN is 0x05. CAN is not used by default. |
|  | Block sequence numbers | Yes | Indicates whether to use block sequence numbering or not, and defines an offset and a modulo. When block numbering is enabled, block sequence numbers start always from offset and use a modulo increment. i.e. seq. number = offset + (blocks-counter) % modulo Where blocks-counter is incremented after each sent block. Default, value for offset is 0x30. Default value for modulo is 2. By default sequence numbers are not used. |
|  | Maximum block size | Yes | Indicates maximum size of data in a given block in bytes. Notice that this limit does not include added DLE characters and other data-link fields: only payload data. Default value is 512 bytes. |
|  | Checksum options | Yes | Indicates whether to use checksum or not, and which type of checksum to use. We support the following checksum types: - LRC - DRC - CRC16 - LRC + DRC - LRC + CRC16 - DRC + CRC16 Sometimes we must also indicate which bytes are skipped when computing the checksum. Thus, we support the following boolean flags: - LRC skips STX? (true ¦ false) - LRC skips DLE? (true ¦ false) - DRC skips STX? (true ¦ false) - DRC skips DLE? (true ¦ false) - CRC16 skips DLE? (true ¦ false) By default no checksum is used. |
|  | Discard list | Yes | Contains a comma-separated list of all messages which need to be discarded when received from this device, if any. The primary reason, why we use this list is that some devices return unsolicited messages such as “PO” when powered on. This field will be used by 1A-PM to intercept and discard such messages gracefully. By default no message is discarded. |
|  | Prefix discard list | Yes | Contains a comma-separated list of all prefixes used to discard incoming messages starting with those prefixes. By default no prefix is discarded. |
|  | Ack/Nack timeout | Yes | Timeout in milliseconds after which 1A-PM stops waiting for an ACK or NAK from device, and sends the same block again if maximum retries not reached. Default value is 250ms. |
|  | Maximum number of retries | Yes | Indicates how many times 1A-PM must re-send a non acknowledged block of data to the device before signalling a write error. Default value is 3. |
|  | SITA Start / End delimiters (Access keyboards only) | Yes | These two bytes (SOD = Start Of Data / EOD = End Of Data) are used to multiplex data over a single serial port. It is used only for Access keyboards which contain three input devices: MSR, OCR and LSR. Commands and responses exchanged with a physical device in this case look like: <SOD> + <STX> <data> +<ETX><EOD> We use the following defaults from SITA documentation: - MSR SOD = 0x0E and EOD = 0x0F - OCR SOD = 0x1C and EOD = 0x1D - LSR SOD = 0x1E and EOD = 0x1F Notice that no start / end delimiters should be used for other devices. |
|  | Outgoing block header | Yes | Indicates the value of the block header used to send data to a device and whether to use it or not. The header value consists of a comma-separated list of hexadecimal bytes. Outgoing block header is not used by default. However if enabled its default value is: "73,1b,0b,20,20,0f" |
|  | Incoming block header | Yes | Indicates the value of the block header used by the device to send data to 1A-PM and whether to use it or not. The header value consists of a comma-separated list of hexadecimal bytes. Incoming block header is not used by default. However if enabled its default value is: "1b,0b,20,20,00,0f" |
|  | Outgoing end of block trailer | Yes | Indicates the value of the block trailer to add on end of block messages sent to a device and whether to use it or not. The trailer value consists of a comma-separated list of hexadecimal bytes. Outgoing end block trailer is not used by default. However if enabled its default value is: "1b,12". |
|  | Outgoing middle block trailer | Yes | Indicates the value of the trailer to add on middle block messages sent to a device and whether to use it or not. The trailer value consists of a comma-separated list of hexadecimal bytes. Outgoing middle block trailer is not used by default. However if enabled its default value is: "1b,13". |
|  | Incoming block trailer | Yes | Indicates the value of the block trailer used by the device to send data to the PM and whether to use it or not. The trailer value consists of a comma-separated list of hexadecimal bytes. Incoming block trailer is not used by default. However if enabled its default value is: "20". |
|  | EMBH | Yes | Indicates whether to use or not the Enhanced Message and Block Handling (EMBH) mechanism. EMBH is not used by default. |
|  | Middle flag | Yes | Indicates the value of the middle flag present after payload data in outgoing or incoming middle blocks. Middle flag is not used by default. However if enabled its default value is: “40” (i.e. ‘@’). |

Table 3 – 1A-PM configuration options

### Configuration Format

In order to enhance flexibility and easily detect errors, we use XML as the format of 1A-PM configuration file. The current version of our XML schema is provided below. Notice that this is an early version which is likely to change in the future:

And a sample configuration file is provided below:

From the sample configuration above, we can see that most of the parameters can be omitted which simplifies greatly the content of 1A-PM configuration files. Omitted parameters are of three types:

- Parameters which are used for information purposes only such as brand, model and firmware information.

- Parameters which are not used most of the time such as checksum and SOM/EOM.

- Parameters for which good default values can be chosen such as timeouts (ack/nack timeout, keep lock timeout …) and data-link parameters (STX/ETX, Ack/Nack…).

Notice that all default configuration values are located inside the XML schema above, which can be stored as a local file on each workstation or file server, or be hosted by an airline or Amadeus web server.

# Deployment

Deployment is extremely critical to our printing solution, because this concerns thousands of airline workstations, spread between hundreds of airports world-wide. Therefore, special care must be taken to make deployment of software (PE and 1A-PM) and corresponding configuration resources (files) as simple as possible. Most of our architecture and design choices have been made in that sense.

## Print Emulator

This is mostly similar to CUTE platforms; The only difference is the usage of a separate property file when an explicit devices list or a list of devices groups are needed (see section 8.1). In this case, the path to PE configuration file is passed to PE/GUI using the following 1A-Launcher’s command line option:

--pe_conf=<path of PE configuration file >

Again, when only local devices are accessed by PE/GUI no configuration file is needed, because Amadeus print emulator connects - by default - to the local Amadeus peripheral manager on default port.

## Peripheral Manager

### Deliverables

Delivered 1A-PM package should contain:

- 1A-PM executables:

  - Windows™ service exe which acts as a launcher.

  - A set of DLL modules which provide the actual functionality.

- 1A-PM XML schema that is used to verify the format of configuration files.

- A set of batch scripts to install/uninstall and start/stop 1A-PM.

- A sample configuration file that illustrates several 1A-PM options.

- The most up-to-date version of this user guide.

### Packaging

Each version of 1A-PM is packaged as a Windows®-like setup wizard for automated installation/un-installation of 1A-PM. The setup wizard is named as follows:

1A-PMSetup_vX-Y-Z.exe

### Installing/Uninstalling 1A-PM

1A-PM must be installed/un-installed using the supplied setup-wizard either in interactive mode or in silent mode (options /SILENT or /VERYSILENT).

To activate the windows service recovery using silent installation, please use option /RECOVERY=on

Note that the windows service recovery is deactivated by default.

Please note that the use of the windows service recovery could hide issues but would not resolve them. For instance, if the environment is not configured properly (i.e. windows account permissions etc…), it can result in the service to be restarted indefinitely without solving the actual problem.

Error: Reference source not found to 10 below illustrate the installation procedure:

Figure 5 - 1A-PM setup wizard: screenshot 1 of 6.

Figure 6 - 1A-PM setup wizard: screenshot 2 of 6.

Figure 7 - 1A-PM setup wizard: screenshot 3 of 6.

Figure 8 - 1A-PM setup wizard: screenshot 4 of 6.

Figure 9 - 1A-PM setup wizard: screenshot 5 of 6.

Figure 10 - 1A-PM setup wizard: screenshot 6 of 6.

### Custom logging options (optional)

IMPORTANT:

Because of a limitation in the way Windows® services access environment variables, a workstation reboot is required for the new values to take effect.

- Log directory

At start-up, 1A-PM learns the location of its logging directory from the following environment variable:

1A-PM_LOG_DIR

Example: 1A-PM_LOG_DIR=C:\Temp

When such variable does not exist or is empty, 1A-PM create its log file(s) by default in the following directory:

<1A-PM root directory>\log

Where the root directory is wherever 1A-PM is installed (default is C:\1a-pm). If either the configured or default log directory does not exit it will be created automatically by 1A-PM.

- Log level

The logging level can be configured using the following optional environment variable:

1A-PM_LOG_LEVEL

All possible values, in increasing restrictive order, are:

ALL, DEBUG, INFO, WARNING, ERROR and NONE

Example: 1A-PM_LOG_LEVEL=ALL

Notice that the keywords above are case sensitive (i.e. “all” and “All” are not valid values for example). If no log level is configured or the value is not in the list above, 1A-PM logs by default at INFO level, which means that only information, warning and error messages are logged.

- Log size

1A-PM uses two cyclic (rolling) files “1a-pm.log” and “1a-pm.log.OLD” to store its logs. By default each of these files has a maximum size of 1MB. However this can be overridden using the following environment variable:

1A-PM_LOG_SIZE

Example: 1A-PM_LOG_SIZE=10485760

The example above changes this size to 10MB (10485760 = 10 x 1024 x 1024) per file. Thus the total logs size is 20MB.

### Custom configuration file location (optional)

1A-PM learns the location of its configuration file from the following environment variable:

1A-PM_CONF_FILE

When such variable does not exist or is empty, 1A-PM expects its configuration file to be located in:

<1A-PM root directory>\conf\pm_conf.xml

### Start delay (optional)

To avoid problems with the LH USB driver dependency, the PM can wait for a while until it is ready. The start delay (in milliseconds) can be set using the following environment variable:

1A-PM_START_DELAY

Example: 1A-PM_START_DELAY=1000 (to wait one second). The default value is 0.

### Custom TCP port number (optional)

1A-PM learns the TCP port number on which client requests can be received from its configuration file. Notice the tcp-port attribute of configuration tag in provided sample configuration file (8.2.2). If no custom TCP port is defined; then the default value (40000) from the XML schema is used.

### Workstation ID (optional)

To identify in a unique way any device connected to the peripheral manager the workstation ID must be specified to the peripheral manager. This workstation ID is set in the configuration file (workstationID attribute of the configuration tag).

The workstation ID can also be retrieved from an environment variable. To do that set the workstationID attribute to $XYZ where XYZ is the environment variable.

Sample: to retrieve the workstation ID from the environment  variable WKS_ID:

<configuration xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:noNamespaceSchemaLocation="pm_schema.xsd" workstationID="$WKS_ID">

Note that the workstation ID is mandatory for RES and optional for DCS.

### Disable the mode command

Some problems were found while testing the 2D LSR. This latter is using a USB to Com emulator which converts the USB port into a virtual COM9 port for instance. There seems to be problems with this driver. Once installed, the mode.com command line does not work anymore. It stalls on the virtual com port. The problem faced with the 1A-PM is that it relies on this mode.com shell command.

Nevertheless in order to solve this potential problem encountered with some devices, a new property has been put in place to specify whether or not a device supports the mode command. By default this property remains true as before. But for the 2D LSR it will have to be set to false.

Note: the property is applying for a COM port, not for a specific device.

The mode supported property is set in the configuration file (mode-supported attribute of the serial-port tag).

Sample:

<device type="LSR" index="0">

<hardware-info brand="2DLSR" model="IT 4600" />

<settings sharable="true" bidirectional="false" …………>

<serial-port ………… mode-supported="false"/>

……………………

</settings>

</device>

### Enforcing 1A-PM version (optional)

By default, we provide a robust versioning mechanism with fallback functionality, to increase 1A-PM’s tolerance to errors. Thus, at start-up 1A-PM’s launcher interrogates Amadeus Version Server to check whether the current version is valid, downgraded or invalid. If the current version is valid, then its corresponding DLLs are loaded dynamically. If it is downgraded, the fallback version returned by the server is loaded instead. Finally, if it is invalid 1A-PM exists with an error message. Notice that when Amadeus Version Server is unreachable (either because it is down or because of a network issue), the current version is loaded by default.

In order to bypass the versioning mechanism above, you can force 1A-PM to load any available version by defining the following environment variable:

1A-PM_VERSION

Example: 1A-PM_VERSION=1-1-0

### Starting/Stopping 1A-PM

Since 1A-PM is implemented as a Windows service; it can be configured to start automatically when a workstation is powered-on. It can also benefit from Windows fault-tolerance features such as automatic restart after a fatal error. A maximum number of retries must be used to avoid eternally restarting a miss-configured 1A-PM.

Finally, 1A-PM can be started or stopped manually from console as follows:

net start | stop 1A-PM

Notice that you can also use provided start.bat, restart .bat and stop.bat scripts.

# Conclusion

The key points of this 1A-PM user guide are summarized below:

- 1A-PM targets native DCS and RES platforms.

- 1A-PM is fundamentally similar to other CUTE peripheral managers.

- 1A-PM provides port adaptation and sharing logic for native serial devices.

- 1A-PM is implemented as a Windows™ service.

- 1A-PM communicates with local and remote PE clients using TCP/IP.

- 1A-PM uses a locally accessible configuration file per workstation.

- 1A-PM’s configuration file is an XML document which must comply with our schema.

# Glossary

1A-PM 	– Amadeus Peripheral Manager

API 	– Application Programming Interface

ATB 	– Automated Ticket and Boarding pass

BTP 	– Bag Tag Printer

CUTE	– Common Use Terminal Equipment

DCS 	– Departure Control System

DGR	– Dumb boarding Gate Reader

GUI	– Graphical User Interface

LSR	– Laser Scan Reader

MSR	– Magnetic Swipe Reader

OCR	– Optical Character Reader

PE 	– Amadeus Print Emulator

PM 	– Peripheral Manager

RES 	– Airline reservation system

# Appendix

## How to install the generic text spooler?

### Generic printer attached to the LPT port

1) Add a new “local printer attached to this computer” (in the Control Panel).

2) Select the LPT1 port

3) Select the generic text spooler (in the generic manufacturer)

### Generic printer attached to an USB port

You just have to configure the “Generic / Text Only” spooler, using USB port.

Here is the procedure to install the generic text driver with an USB printer:

1) Install the driver of the USB printer

2) In the control panel, you should then have a new spooler for this printer

3) Right click on this spooler and click properties

4) In the Ports tabs, look what is the used port (for instance in this case it is “DOT4_001”):

5) Close the properties

6) Delete the spooler of the USB printer (in the control panel, fax and printers)

7) Add new printer (local printer attached to this workstation)

8) Select the proper port in the list (the one you looked for in step 4)

9) Select the Generic / Text Only driver

Printer is now ready to be used in raw text mode with the 1A-PM. You can do a simple test, printing from notepad on the Generic / Text Only spooler.

Only generic printers with proper generic text driver installed are supported on Windows spooler.

Only generic printers are supported on LAN (i.e. general purpose network printers).

Default port 40000 is provided only for illustration only, should any other value be more convenient, please let us know.

MSDN: http://msdn.microsoft.com/library/default.asp?url=/library/en-us/devio/base/buildcommdcb.asp